/*

* Automatic Single-Axis Solar Tracker System

* Uganda Institute of Information and Communications Technology (UICT)

* Department of ICT and Engineering

*

* This code controls a single-axis solar tracker using LDR sensors,

* servo motor, voltage/current sensors, LCD display, RTC, SD card,

* and Bluetooth module.

* Hardware Components:

* ESP 32 WROOM Microcontroller

* - 2 x LDR sensors (Left and Right)

* - Servo motor (for panel rotation)

* - Voltage sensor (for panel voltage monitoring)

* - Current sensor (for panel current monitoring)

* - LCD display (16x2 with I2C)

* - RTC DS3231 (for timekeeping)

* - SD card module (for data logging)

* - HC-05 Bluetooth module (for wireless monitoring)

*/

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <Servo.h>

#include <RTClib.h>

#include <SPI.h>

#include <SD.h>

#include <SoftwareSerial.h>

#include <math.h>

// ============================

// PIN DEFINITIONS

// ============================

// LDR Sensors

#define LDR_LEFT_PIN   A0

#define LDR_RIGHT_PIN  A1

// Voltage and Current Sensors

#define VOLT_PIN       A2

#define CURR_PIN       A3

// Servo Motor

#define SERVO_PIN      9

// SD Card Module

#define SD_CS_PIN      10

// Bluetooth Module (SoftwareSerial)

#define BT_RX_PIN      0   // RX of Arduino to TX of Bluetooth

#define BT_TX_PIN      1   // TX of Arduino to RX of Bluetooth

// ============================

// CONSTANTS

// ============================

// Servo limits

#define SERVO_MIN      0

#define SERVO_MAX      180

#define SERVO_CENTER   90

#define SERVO_STEP     1

// LDR dead zone (prevents oscillation)

#define LDR_THRESH     50

// Sampling interval (milliseconds)

#define LOOP_MS        200

// Voltage divider resistors (for voltage sensor)

#define R1             30000.0f

#define R2             7500.0f

// Current sensor calibration (ACS712)

#define ACS_SENS       0.045f   // Sensitivity: 45mV/A for 20A module

#define ACS_ZERO       2.5f     // Zero current voltage (2.5V)

// ============================

// OBJECTS

// ============================

LiquidCrystal_I2C lcd(0x27, 16, 2);

Servo srv;

RTC_DS3231 rtc;

SoftwareSerial bluetooth(BT_RX_PIN, BT_TX_PIN);

// ============================

// GLOBAL VARIABLES

// ============================

// System mode

enum Mode { AUTO, MANUAL_BT };

Mode mode = AUTO;

// Current servo angle

int curAngle = SERVO_CENTER;

int prevAngle = -1;

// SD card status

bool sdOK = false;

// Bluetooth connection status

bool btConn = false;

// ============================

// HELPER FUNCTIONS

// ============================

// Read voltage from solar panel

float readVoltage() {

int reading = analogRead(VOLT_PIN);

float voltage = reading * (5.0f / 1023.0f) * ((R1 + R2) / R2);

return voltage;

}

// Read current from solar panel

float readCurrent() {

int reading = analogRead(CURR_PIN);

float voltage = reading * (5.0f / 1023.0f);

float current = (voltage - ACS_ZERO) / ACS_SENS;

return current;

}

// Move servo to specified angle

void moveServo(int angle) {

curAngle = constrain(angle, SERVO_MIN, SERVO_MAX);

srv.write(curAngle);

delay(50);

}

// ============================

// AUTO TRACKING LOGIC

// ============================

void autoTrack() {

// Read both LDRs

int leftValue = analogRead(LDR_LEFT_PIN);

int rightValue = analogRead(LDR_RIGHT_PIN);

// Calculate difference

int diff = leftValue - rightValue;

// If difference exceeds threshold, move servo

if (abs(diff) > LDR_THRESH) {

if (diff > 0) {

// More light on left -> rotate right

moveServo(curAngle + SERVO_STEP);

} else {

// More light on right -> rotate left

moveServo(curAngle - SERVO_STEP);

}

}

}

// ============================

// BLUETOOTH HANDLER

// ============================

void handleBluetooth(float v, float i) {

if (bluetooth.available()) {

String cmd = bluetooth.readStringUntil('\n');

cmd.trim();

btConn = true;

if (cmd.equalsIgnoreCase("AUTO")) {

mode = AUTO;

bluetooth.println("Mode: AUTO");

}

else if (cmd.equalsIgnoreCase("MANUAL")) {

mode = MANUAL_BT;

bluetooth.println("Mode: MANUAL");

}

else if (cmd.startsWith("SET:")) {

if (mode == MANUAL_BT) {

int angle = cmd.substring(4).toInt();

moveServo(angle);

bluetooth.print("OK: ");

bluetooth.println(curAngle);

} else {

bluetooth.println("ERR: Switch to MANUAL mode first");

}

}

else if (cmd.equalsIgnoreCase("STATUS")) {

bluetooth.print("Angle: ");

bluetooth.print(curAngle);

bluetooth.print(" | V: ");

bluetooth.print(v);

bluetooth.print(" | I: ");

bluetooth.println(i);

}

else if (cmd.equalsIgnoreCase("HELP")) {

bluetooth.println("Commands:");

bluetooth.println("  AUTO     - Enable auto tracking");

bluetooth.println("  MANUAL   - Enable manual control");

bluetooth.println("  SET:XX   - Set angle (XX = 0-180)");

bluetooth.println("  STATUS   - Show current status");

bluetooth.println("  HELP     - Show this menu");

}

else {

bluetooth.println("ERR: Unknown command");

}

}

}

// ============================

// DATA LOGGING

// ============================

void logData(float v, float i) {

DateTime now = rtc.now();

if (sdOK) {

File dataFile = SD.open("TRACKER.TXT", FILE_WRITE);

if (dataFile) {

dataFile.print(now.year());

dataFile.print("-");

dataFile.print(now.month());

dataFile.print("-");

dataFile.print(now.day());

dataFile.print(" ");

dataFile.print(now.hour());

dataFile.print(":");

dataFile.print(now.minute());

dataFile.print(":");

dataFile.print(now.second());

dataFile.print(",");

dataFile.print(curAngle);

dataFile.print(",");

dataFile.print(v);

dataFile.print(",");

dataFile.println(i);

dataFile.close();

}

}

}

// ============================

// LCD DISPLAY UPDATE

// ============================

void updateLCD(float v, float i) {

lcd.setCursor(0, 0);

// Display mode

if (mode == AUTO) {

lcd.print("AUTO ");

} else {

lcd.print("MANUAL");

}

// Display angle

lcd.print(" ANG:");

lcd.print(curAngle);

lcd.print(" ");

// Display voltage and current

lcd.setCursor(0, 1);

lcd.print("V:");

lcd.print(v, 1);

lcd.print(" I:");

lcd.print(i, 2);

lcd.print("A ");

}

// ============================

// SETUP FUNCTION

// ============================

void setup() {

// Initialize serial communication

Serial.begin(9600);

bluetooth.begin(9600);

Wire.begin();

// Initialize LCD

lcd.init();

lcd.backlight();

lcd.clear();

lcd.print("Solar Tracker");

lcd.setCursor(0, 1);

lcd.print("Initializing...");

delay(1000);

// Initialize RTC

if (!rtc.begin()) {

lcd.clear();

lcd.print("RTC Error!");

Serial.println("RTC initialization failed!");

while (1);

}

Serial.println("RTC initialized.");

// Initialize SD Card

if (!SD.begin(SD_CS_PIN)) {

lcd.clear();

lcd.print("SD Card Error!");

Serial.println("SD Card initialization failed!");

sdOK = false;

delay(2000);

} else {

sdOK = true;

Serial.println("SD Card initialized.");

}

// Initialize Servo

srv.attach(SERVO_PIN);

moveServo(SERVO_CENTER);

delay(600);

// Display ready message

lcd.clear();

lcd.print("Mode: AUTO");

Serial.println("Solar Tracker System Ready.");

}

// ============================

// MAIN LOOP

// ============================

void loop() {

// Read sensors

float voltage = readVoltage();

float current = readCurrent();

// Auto tracking logic

if (mode == AUTO) {

autoTrack();

}

// Handle Bluetooth commands

handleBluetooth(voltage, current);

// Log data if angle changed

if (curAngle != prevAngle) {

logData(voltage, current);

prevAngle = curAngle;

}

// Update LCD display

updateLCD(voltage, current);

// Wait before next loop

delay(LOOP_MS);

}